Information about the 3dMax plugin to export .dts shapes:

( author unknown )

There are two projects: dtsexp and seqhlp.  Each makes a .dlo file that

goes in the 3dsmax\plugins directory.  Dtsexp is the main piece of the

exporter that converts the MAX shape into 3space format and saves it.

Helpers has a couple of "helper" objects defined: sequence and

transition.  These definitions allow the artist to create these objects

and attach data to them.  The Dtsexp plugin later finds these objects

and retrieves the data.

Animation in a MAX file gets done as one long series, while 3space

supports playing different sequences simultaneously.  The artist uses

the sequence objects to mark start and end points on the MAX timeline.

To avoid conflicts in sequences that might be meant to play at the same

time, I made certain assumptions.  Generally, only the types of

animation where change occurs during a sequence are added as keyframes.

For example, in a walk sequence where the head node is visible

throughout the sequence, no visibility keyframes are exported.  This

allows another sequence to control the visibility of the head node, and

be played at the same time with no conflict.  The downside to this is

that sometimes an artist will turn the visibility off for a node and

keep it off for an entire sequence, expecting that that sequence will

turn off the node when played.  But since no visibility change is

detected, no vis keyframes get exported.  There is a force_vis flag on

the sequence which causes the vis keyframes to be exported regardless,

but this affects all nodes.

Transform animation has keyframes that can be viewed in the MAX track

info view.  The exporter spits out a keyframe for each of these that

occur during a sequence.  For morph animation, we use the cel frame rate

that's entered by the artist into the sequence helper object to figure

the keyframes. The exporter samples the mesh on a node at the specified

frame rate, comparing each sampled frame against the last frame it has

to see if it's worth exporting.  Also, visibility and material changes

are scanned for in this manner and keyframes for those are exported if

changes occur.  Multi-part shapes such as the robots in Es3 generally

don't have mesh changes, so each mesh just has one frame, while they

have several transform keyframes.  The original Fear character on the

other hand has no transform changes, just mesh changes.  There are

shapes with both types at once, and shapes where one sequence has morph

frames for a mesh on a node, while another sequence meant to be played

simultaneously has transform keyframes for the same node.

There are two types of material animation.  The first type is where the

artist animates the UVW mapping gizmo, which causes the texture

coordinates to move during a sequence.  This is similar to a morph

change, just on texture coords instead of vertices.  This type is

scanned for just like a morph change (relies on cel frame rate).  For

each frame of this animation that exists, every texture vertex in the

mesh is spit out, even if only one coord changes (just like morph

animation).  The second type is .ifl animation.  If the artist points

the material to an .ifl file instead of a .bmp or .tga file, MAX will

parse the .ifl file (as a text file), and decide which material to use

at a given time based on what the .ifl file says.  These changes are

global to all faces that reference this material.  For each material

that points to an .ifl file, I add every material in the .ifl file that

occurs during a sequence to the material list, and save a table along

with each sequence so 3space knows how to remap to get the proper

material at the proper time.  This is done as a post-process in

ShapeBuilder::makeShape().

DTSEXP.DLO:

SceneEnum is the main file for following the flow.  The first thing that

happens is that the callback function at the bottom of the file gets

called for each node in the scene.  Then getNodes() gets called which

grabs the bounding box and then attaches the other top nodes to it.  We

assume that nodes at the top of the tree (nodes with parent "root") that

have meshes attached to them are either the bounds box (which has to be

named "bounds") or are detail nodes, probably with a heirarchy of nodes

under them.  The name of the detail nodes should have a number at the

end of them which is used to set the pixel size for detail selection.

As each node is processed in getNode(), it looks for transform

animation and/or visibility/morph/material animation, then processes its

child nodes.  Notes: If a node's name begins with "dummy", the vis/morph

animation is ignored.  This allows for placeholder nodes to be put in a

shape without causing MAX to export the geometry of the placeholder

piece.  If a node's name begins with "always", it is attached to a

special node of the shape (the alwaysNode), which means it gets drawn

regardless of the detail that was selected.  A good use of this is

plugin objects, like the gun in Fear, because it does it's own

detailing, and we don't want to duplicate a gun in each detail level.

If you have to combine "dummy" and "always", use "dummyalways".

"always" nodes do not have to be connected to a hierarchy that is

exported...they can be floating somewhere that would normally be skipped

over.

Meshbuilder.cpp has the code that deals with morph frames of a mesh.  It

converts from MAX's mesh format to 3space's, then multiplies the points

by a passed matrix, then compares them to the last frame it grabbed

(unless there isn't one yet).  Also computes normals for the verts,

which gets a little complicated.  Flat shaded polys need the normal of

their first vertex to be pointing normal to the surface, while gouraud

polys expect their vertex normals to be the average of the normals of

the gouraud faces that share that vertex.  Sometimes a vertex must be

duplicated if two adjacent faces both need it pointing different

directions.  Adjacent gouraud polys that are from different shading

groups do not average their normals with each other, so that also causes

vertex duplication.

Shapebuilder.cpp has a class that basically collects the data until the

nodes are all processed, then it uses the data to build the TSShape that

gets written out.	 Also has code to accumulate the references to

materials so it can write out a material list file (gets written out as

part of the .dts file).

The materials have a few details to mention.  A bitmap file with a '@'

character in the name means to set the transparent flag in the material.

If the material filename is a ".tga", I convert the name to ".bmp", and

set the translucent flag.  It is assumed that somebody will convert the

.tga file to a .bmp and palette file before this is used in an app.

Material filenames with ".ifl" are treated differently depending on what

time the material is referenced.  ".ifl" files are ascii files that have

a list of .bmp or .tga files which are meant to be used for material

animation.  Each entry can also have a number following it that is the

hold count in frames (how long to wait before switching to next

material).  All materials in the shape that point to the .ifl get

animated at once.  When I scan the shape for morph animation, I also get

the current material (evaluate the .ifl into the .bmp that should be

used at the current time).  If it has changed I output a keyframe for

material animation, and that material will then be put in the material

list.  Not necessarily all materials listed in an .ifl file will end up

in the material list (only if they occur during a sequence).

Misc Notes:

- We decided against having an object offset, so this is always

pre-multiplied into the mesh, and set to identity.  We also decided that

scaling in any transform was bad, since it complicates things at

run-time, so when I encounter scale in a transform, I push it down to

the children of that node, and clear it to 1.0's.  So it get's collapsed

down into the meshes.  One thing that's assumed here is that there won't

be any animated scaling taking place, because I compute the scale of the

transform once, passing that down to the sub-hierarchy, but I clear the

scale of all the transform keyframes.

- The type of shapes we used at one time in Fear required some special

cases in the exporter.  The old Fear character was a mesh that was

generated from a Character Studio skeleton (current character is

separate parts, like an ES shape).  We export the mesh as morph

animation, and ignore the skeleton completely. The mesh and the skeleton

end up both being linked to a special "VICON" node, which has a

transform that kind of represents the hips (base transform of the

character).  The first hack is that the mesh node isn't at the top level

of the scene, so it normally wouldn't be exported (it's parent nodes

don't have meshes, so they get skipped). So I also export hierarchies

that are named "VICON" (this node isn't necessarily on the root level

either).  Second hack is that I want the transform info in the VICON node

that I'm skipping over included in the mesh's motion. And I don't want it

included as transform animation, because that would end up being

interpolated smoothly, while the mesh is sampled and doesn't tween. The

combination of these two types looks dumb. So I have to grab the

transform and multiply all the mesh points by it before exporting them.

This is what is going on inside the #if VICON_MUL clauses.  Third problem

is that there can be only one VICON node, yet we may want multiple detail

levels.  So I take each mesh node attached to the VICON, and make that a

detail node, attaching the rest of the nodes in the VICON hierarchy with

matching detail numbers to it.  So one pass through the hierarchy is

made for each mesh node (detail) linked to the VICON.

- I had some problems with the assumption that sequences that don't

affect visibility don't export any vis keyframes.  Sometimes an artist

will make a sequence where a node's visibility is off for the entire

sequence, but want keyframes to be exported for that node.  The

sequence's force_vis flag could be set, but that would export visibility

keyframes for all nodes, which may not be what you want.  The solution

is to make the node default to invisible when no threads are running

sequences that affect the visibility.  To let 3space know of this

intent, name the node something beginning with "hide".

- Sequences have their start and end marked with keyframes in the track

view, but there can also be intermediate keyframes which control the

frame rate (used for morph animation sampling).  The start key could be

60 fps, then perhaps you might want less resolution for the second half

of the sequence, so you put a 30 fps key frame in the middle.

HELPERS.DLO (project is called seqhlp):

The main code here is in sequence.cpp.  This just has a bunch of data

defining how the transition and sequence helper objects behave.  I don't

know really how everything works, I just used the code as an example

when I wanted to add fields of my own.  The main thing to remember here

is that it doesn't work in the debugger (although it looks like it at

first) which explains all the commented out printf's.

BUILDING STUFF:

I use the VC 5.0 IDE to make the .dlo files, but I use the command line

compiler to make the darkstar libs that the 2 projects use.  You don't

need all the DS libs, just core, ml, gfxio, and ts3.